Custom Bot Using ROS2 nav for food serving

PROJECT OVERVIEW

The French Door Café aims to streamline operations by introducing a robotic butler to handle food deliveries during peak hours. This project presents a custom-built mobile robot that autonomously navigates between the kitchen and customer tables using ROS 2 and the Nav2 stack.The robot starts from a home position, picks up orders from the kitchen, delivers them to Table 1, 2, or 3, and returns to its base

SYSTEM ARCHITECTURE

## 1. Core Package Files

These files define the package and its dependencies.

- package.xml: Describes the package metadata, dependencies (e.g., rclpy, nav2, gazebo_ros), license, author info.

- setup.py / setup.cfg: Python packaging files used to make this a valid installable ROS 2 Python package.

## 2. URDF and Robot Description (urdf/)

This folder defines the robot's physical and visual structure using XACRO and URDF.

- turtlebot3_burger.urdf: URDF for the base TurtleBot3 (used as a reference or base).

- b_bot.xacro: The main robot description written in XACRO, a macro-enabled XML language to simplify URDF files.

- materials.xacro: Contains color/material definitions used in the robot’s model.

- b_bot.trans: Likely includes static transform definitions.

- b_bot.gazebo: Gazebo-specific extensions like plugins for sensors or joints.

- gazebo_control.xacro: Defines ROS control interfaces and Gazebo plugins for controlling joints/wheels.

## 3. Launch Files (launch/)

These launch files orchestrate different simulation and real-world tasks.

- gazebo.launch.py: Launches the Gazebo simulator with the robot.

- slam.launch.py: Starts SLAM (e.g., slam_toolbox) to build a map of the environment.

- navigation2.launch.py: Starts the full Nav2 stack (planner, controller, behavior tree, etc.).

- display.launch.py: Likely opens RViz with the robot model and config.

## 4. Configuration Files (config/)

Config files for RViz and Nav2 behavior.

- nav2_params.yaml / nav2_params2.yaml: Parameters for the Navigation2 stack (planner settings, controller plugins, recovery behavior).

- mapper_params_online_async.yaml: SLAM parameters for async online mapping.

- display.rviz: RViz configuration to load a specific visualization layout.

## 5. Behavior Scripts (b_bot_description/)

These Python scripts implement robot behavior logic.

- send_goal.py: Sends navigation goals to the NavigateToPose action server.

- order_timeout.py: Likely handles timeouts or failed delivery goals.

- follow_path.py: Possibly used to execute custom path-following logic.

- __init__.py: Makes the directory a valid Python module.

## 6. Tests (test/)

Standard ROS 2 testing framework support.

- test_copyright.py: Checks if source files have copyright.

- test_flake8.py: Enforces PEP8 style via Flake8.

- test_pep257.py: Ensures docstring style compliance.

## 7. Meshes and Models

Used for realistic visualization in RViz and Gazebo.

- meshes/: STL 3D models for visualization in Gazebo/RViz.

  - Includes parts like lidar_1.stl, base_link.stl, left_wheel_1.stl, etc.

- model/turtlebot3_burger/: SDF and config files for the TurtleBot3 simulation model.

  - model.sdf, model-1_4.sdf, model.config define the model’s appearance and behavior in Gazebo.

## 8. Worlds (worlds/)

Gazebo world definitions.

- cafe_world.sdf / coffee_shop_world.sdf: Simulated environments for the robot to operate in.

## 9. Maps (maps/)

Pre-built map used for localization and navigation.

- map.pgm: Image file of the map.

- map.yaml: Metadata describing the origin, resolution, and thresholds of the map image

Launch Instructions:

- Build & source the workspace

- To launch gazebo world:

- ros2 launch b_bot_description gazebo.launch.py

- To launch nav2:

- ros2 launch b_bot_description navigation2.launch.py

- To run delivery script:

- ros2 run b_bot_description order

- To give table num as command, open new terminal and paste

- ros2 topic pub -1 /order std_msgs/msg/String "data: 'table1,table2,table3'"

- To give another order after the robot returns to home

- kill the delivery script and re-run it.

## Robot Commands

| Command | Description |
| --- | --- |
| table1 | Deliver to Table 1 |
| Table1,table2,table3 | Multiple table delivery |
| yes | Confirm delivery |
| cancel | Cancel and go home |
| c1, c2, c3 | Cancel a specific table |

Flow Diagram:

Output: